Optimally-Discriminative
Voxel-Based Analysis

User Manual

Tianhao Zhang

November 28, 2012

Contents

1.	Introduction	1

2.	Preparation Steps	2

2.1  Supported Image Formats	2

2.2  Preprocessing	2

2.3  Preparation of the Subjects List	2

3.	Performing the group analysis	3

4.	References	6

## Introduction

This software package implements the Optimally-Discriminative Voxel-based Analysis (ODVBA) [1], which is used to determine the optimal spatially adaptive smoothing of images, followed by applying a voxel-based group analysis.

Voxel-based Analysis and Statistical Parametric Mapping (VBA-SPM) [2] of imaging data have offered the potential to analyze structural and functional data in great spatial detail, without the need to define a priori regions of interest (ROIs) and assumptions. Gaussian smoothing of images is an important step in VBA-SPM since it accounts for registration errors and integrates imaging signals from a region around each voxel being analyzed. However, it has also become a limitation of VBA-SPM based methods, since it is often chosen empirically, non-optimally, and lacks spatial adaptivity to the shape and spatial extent of the region of interest.

ODVBA provides a mathematically rigorous framework for determining the optimal spatial smoothing of structural and functional images, prior to applying voxel-based group analysis.  In order to determine the optimal smoothing kernel, a local discriminative analysis, restricted by appropriate non-negativity constraints, is applied to a spatial neighborhood around each voxel, aiming to find the direction best highlights the difference between two groups in that neighborhood. Since each voxel belongs to a large number of such neighborhoods, each centered on one of its neighboring voxels, the group difference at each voxel is determined by a composition of all these optimal smoothing directions. Permutation tests are used to obtain the statistical significance of the resulting Optimally-Discriminative VBM (ODVBA) maps.

The software package exploits some randomly selected data from the Alzheimer’s Disease Neuroimaging Initiative (ADNI) [3] as an example to show the usage of the ODVBA.

## Preparation Steps

### 2.1  Supported Image Formats

The software supports the ANALYZE 7.5 and NIfTI formats, where the voxel type of the scalar input images is either float (DT_FLOAT) or short (DT_SIGNED_SHORT).

### 2.2  Preprocessing

The input images can be any type of tissue density maps, e.g., RAVENS map.

Downsampling images to 2mm×2mm×2mm is required.

Gaussian smoothing is not needed. For images with potential noise, a very small Gaussian smoothing (FWHM = 2mm) can be utilized before conducting group analysis.

### 2.3  Preparation of the Subjects List

For input convenience, one subjects list file in below format is used by all programs, with assuming the values of patients (upper) < that of controls (lower).

<Number of Patients M> <Number of Normal Controls N>

/base/path

<Path to image of patient 1>

.

.

.

<Path to image of patient M>

<Path to image of control subject 1>

.

.

.

<Path to image of control subject N>

On the first line, the number of patients is given followed by the number of control subjects, where these two numbers are separated by one or more whitespace characters. The second line specifies the base path to image files either absolute or relative to the directory in which the subjects list files is located. The subsequent lines contain the paths to the image files, first the ones of the patients and second the ones of the control subjects. These paths can be given either relative, in which case they are considered relative to the base path specified on line two, or absolute. Note that the filename extensions .nii, .hdr or .img can be omitted.

A sample subject list file from data retrieved as part of the Alzheimer’s Disease Neuroimaging Initiative (ADNI) is shown below. This sample file contains 100 subjects.

50 50

./dir

003_S_1059-2006-11-09_RAVENS_GM_ds_le

011_S_0010-2005-11-07_RAVENS_GM_ds_le

011_S_0053-2005-11-14_RAVENS_GM_ds_le

.

.

.

003_S_0907-2006-09-11_RAVENS_GM_ds_le

011_S_0002-2005-08-26_RAVENS_GM_ds_le

011_S_0005-2005-09-02_RAVENS_GM_ds_le

.

.

.

## Performing the group analysis

Run ODVBA within the cluster environment at SBIA either serially in an interactive session or as parallel MPI job submitted to the batch-queuing system SGE. By default, if no number of CPUs is specified and this script is submitted to SGE via qsub, it will request 6 slots and the permutation tests will be performed by 6 parallel MPI processes. A different number of parallel processes can be specified using the "-pe openmpi <#CPUs>" option to qsub. If the number of CPUs requested is less or equal to 1, the serial command odvba is executed instead of the MPI command odvba-mpi.

The command messages of ODVBA are by default written to the file:

${HOME}/sge_job_output/odvba.\$JOB_ID.log

Please see below for details on the usage of the odvba[-mpi] commands, where odvba-sbia should be used at SBIA in place of these commands.

COMMAND:

odvba-sbia

OPTIONS

Required arguments:

<subjects_list>

Text file which lists the NIfTI-1 image data files of the  input subjects.

Optional arguments:

--index <file>

Name of the pre-computed index file.

--index-out <file>

Name of the file to write the computed index information to.

--ni <file>

Name of the neighborhood indices file. If none of the other neighborhood relatedoptions (--ni-*) are given, the  generated neighborhood indices are written to the specified file. Otherwise, these are read from the given file which must have been generated before using odvba-ni.

--ni-size <double>

Size of neighborhood in mm. Required and used only if neighborhood indices file not provided. Suggested value: 15.

--ni-num <int>

Number of neighborhoods. At most, the neighborhood around each non-zero voxel can be considered, i.e., n <= m, where m is the number of non-zero voxels. Suggested value: 10000.

--ni-voxels <int>

Number of voxel in each neighborhood. Suggested value: 400.

--perms-num <int>

The number of permutations to test. Suggested value: 1000-5000.

--phi <double>

Exponent phi of discrimination degree (cf. Eq.(11) in Ref. [1]). Suggested  value: 2.

--maps <file>

Name of file to which the group analysis results are written. If this option is not specified, only the final image of p-values is output.

--perms <file>

If the number of permutation tests specified by the option --perms-num is zero, the pre-computed permutations stored in the given file are used. Otherwise, the generated random permutations used for the permutation tests are written to the specified file. If this option is present but not --perms-num, --perms-num is assumed to be zero, hence, the permutations will be read from the specified file. This option is used for regression testing, in particular.

--p-image <nifti>

Filename prefix of the output volume of p-values which is output as NIfTI-1 image with voxel type DT_FLOAT.

--srand-time

Use the execution time to initialize the pseudo-random number generator. If this option is not given, a fixed seed is used to initialize it which ensures identical results               whenever this program is run on the same machine with the same input images and parameter settings. Otherwise, the results may differ between executions.

EXAMPLE:

qsub -M your.account@uphs.upenn.edu -m b -m e -m a -pe openmpi 30 odvba-sbia -v --perms-num 1000 --phi 2 --ni-num 10000 --ni-voxels 400 --ni-size 15 --index-out index.txt your.path/example/subjects.txt

Explanation: Run 1000 permutation tests after generating 10000 random neighborhoods with 400 randomly selected voxels within windows of size 15mm. The index of the computing voxels is written into index.txt. The final image of p-values is written into p.nii.gz.

## References

- T. Zhang, C. Davatzikos, ODVBA: Optimally-Discriminative Voxel-Based Analysis, IEEE Transactions on Medical Imaging, vol. 30, no. 8, 1441-1454 (2011)

- Ashburner, J., Friston, K.J.: Voxel-based morphometry-the methods. Neuroimage, 11(6) 805–821 (2000)

- Alzheimer’s Disease Neuroimaging Initiative: www.loni.ucla.edu/ADNI