# White matter lesion segmentation from multi-spectral brain MRI

## Introduction

White matter lesions (WML) are brain abnormalities that appear in different brain diseases, such as multiple sclerosis (MS), head injury, vascular disease particularly related to hypertension possibly diabetes, and some forms of dementia. Their incidence also increases with normal aging. MRI is routinely used as surrogate in the study of WMLs, as MRI signal changes reflect certain aspects of the underlying brain pathology. Out of the many available MRI acquisition protocols, T1-w and T2-w, PD, and FLAIR are among the most commonly used to evaluate WML load in the brain. Computer analysis methods have started to complement expert-readings of these images, as they may improve throughput and consistency, in addition to providing more accurate quantitative measures of WML. Computer analysis is even more critical in longitudinal studies that involve relatively small changes in WML, thereby rendering it advantageous, if not necessary, to use unbiased computer-assisted segmentation methods to detect WMLs and assess their longitudinal change.

We proposed a novel WML segmentation protocol which is a multi-spectral based approach via incorporating information from T1-w, T2-w, PD-w and FLAIR MR Brain images. Numerous techniques in medical image analysis have been utilized in this protocol to make it reach AAA (Automatic, Accurate and Applicable) criteria.

## Method

We propose a novel computer-assisted WML segmentation approach which uses a combination of image analysis and machine learning techniques (Support Vector Machines). Image intensities from multiple MR acquisition protocols, after co-registration, are used to form a voxel-wise attribute vector that helps reduce ambiguity in discriminating lesion and normal tissue during segmentation. In general, there are three steps in our approach, as summarized in the figure below.

- A preprocessing step includes co-registration of different MR images, skull-stripping, intensity normalization, as well as inhomogeneity correction.

- A set of training samples is constructed via manual segmentation by expert readers, and then used to build a classification model via SVM and AdaBoost; this step is applied only once, during training.

- The SVM model is used to perform the voxel-wise segmentation.

- False positive voxels are further eliminated via measuring the distance distribution in Hilbert space, thereby producing final WML segmentation results.

Figure below is a flow chart that illustrates a overview of the WML segmentation algorithm.

## DOWNLOAD

The Software package of WMLSegmentation can be found from:

https://sbia-svn.uphs.upenn.edu/projects/WMLesionSegmentation/

You can download by typing:

svn co https://sbia-svn.uphs.upenn.edu/projects/ WMLesionSegmentation/

There are 6 folders, namely: bin, dependencies, model, sample, scripts, src, respectively.

All the executables are in the folder bin. All dependent executables or scripts are in the folder dependencies, including svm_torch_sgi and svm_test_sgi. Model files used by Support Vector Machines (SVM) and intensity normalization have been put under folder model. Sample subjects for training & testing purpose are in folder sample. All the scripts (Perl and Shell) are in the folder scripts. All source codes are in the folder src.

## Install

If you change the code you can recompile it using the provided makefile under src folder. Type:

make all

## Dependencies

- Files in the model folder

- Executables in the dependencies folder. If you do not move or rename these files, the program should run successfully.

- If flirt is installed, it is used for rigid transformation. If it is not installed, please consult system administrator to install it.

- If N3 package is installed, it is used for correcting inhomogeneity (shading effect). If it’s not installed, please consult system administrator to install it.

## Code structure

If you want to make some modification based on the script, the following introduction will help you to follow how it works. There are several steps in this protocol.

Training:

Usage: WMLSegmentation_train.pl train_list model [options]

-d<int>,<int>,<int>  : dimension of input images in XY plane (default: 256,256,46)

-v<float>,<float>,<float>  : image resolution (default: 0.9375,0.9375,3.0)

-t<float>,<float>    : threshold for TPVF and FPVF (default: 0.9,0.1)

-i<int>              : maximum iteration number (default: 10)

-r<int>              : neighbor size (default: 1)

-M<int>              : use premask or not 0: not use 1: use (default: not use)

-p<string>           : path where this package installed

-w <float>,<float>,<float>,<float> : weighting parameters for t1, t2, pd, flair respectively (default: 1.0,1.0,1.0,1.0)

where:

Train_list: the list of subject used for setting up training model.

Model: SVM model output from the training program.

-d: image dimension of input training subjects with default value to be 256 x 256 x 46.

-v: image resolution (voxel size) of input training subjects with with default value to be 0.9375mm x 0.9375mm x 3.0mm

-t: thresholds for terminate the training process. TPVF is sensitivity threshold with default value to 0.9; FPVF is 1 – specificity threshold with default value to be 0.1;

-i: maximum number of iteration while setting up training model.

-r: the radius of neighborhood in defining Attribute Vector (AV) in mm with default value to be 2mm. For example: if the neighbor size is 5mm x 5mm x 5mm, then this value is 2.

-M: premask for training if just want to focus on certain regions in the image. Default: no use.

-p: the path where this software package installed.

-P: path where to put intermediate result with default to be /tmp.

-w: weighting parameters for each sequence while defining AV with default value to be equal number for all sequences (1.0,1.0,1.0,1.0).

In order to be complete, training stage is written as a stand alone command line script. In practice, training typically only performs once at the very beginning of the project.

Result in training stage will be stored in “model”

Step 1: Build up initial SVM model

Using command “WMLGetSelectedFeature” to extract AVs from training samples and using command “svm_torch_sgi” to generate initial SVM model.

Sample commands:

WMLGetSelectedFeature subjectID.T1.byte.img subjectID.T2.byte.img subjectID.PD.byte.img subjectID.FL.byte.img manual_lesion_mask subjectID.lesion.vec –d256,256 -t0 -m1 -n$average_manual_segmentation_voxel_number –w1.0,1.0,1.0,1.0 –r2

svm_torch_sgi -v -ae -e 0.0002 -t 2 -st 6000 train.example $ARGV[1].0

Step 2: Iteratively refine SVM model

Sub-step 1: Generate evaluation map using SVM model from previous iteration

Using command “WMLTestSingleImageKeepAllFunctionValue” to test training subjects using SVM initially generated.

Sample command:

WMLTestSingleImageKeepAllFunctionValue_NoSmooth subjected.T1.byte.img subjected.T2.byte.img subjected.PD.byte.img subjected.FL.byte.img svm_model subjectID.WML.mask.emap –d256,256 -m1 –w1.0,1.0,1.0,1.0 –r2

Sub-step 2: threshold evaluation map to get binary lesion masks

Using command “OneImgOp” to threshold emap generated in sub-step1 and get binary lesion masks.

Sample command:

OneImgOp subjectID.WML.mask.emap $subs[$i].WML.mask.img –t0.0 -m0 -f1

Sub-step 3: Get difference between automatic lesion mask and manual lesion mask

Using command “WMLGetAutoManualMaskDiff” to get difference mask between manual segmentation and automatic segmentation.

Sample command:

WMLGetAutoManualMaskDiff manual_mask.lst auto_mask.lst err.dat –d256,256,46 –r0.9375,0.9375,3.0

Sub-step 4: if training error is less than desired training error threshold, output SVM model and terminate; else continue.

Sub-step 5: extract AV from the difference mask.

Using command “WMLGetSelectedFeature” to extract additional AVs from difference lesion masks.

Sample command:

WMLGetSelectedFeature subjectID.T1.byte.img subjectID.T1.byte.img subjectID.T1.byte.img subjectID.T1.byte.img subjectID.WML.mask.nonlesion.diff subjectID.nonlesion.vec.$iter –d256,256 -t0 -m1 –w1.0,1.0,1.0,1.0 –r2 -S

Sub-step 6: merge AVs from sub-step 4 with initially extracted AVs and build up new SVM model.

Sample command:

svm_torch_sgi -v -ae -e 0.0005 -t 2 -st 6000 train.example svm_model.$iter

Sub-step 7: goto sub-step 1.

Testing:

Usage: WMLSegmentation_test.pl t1_img t2_img pd_img flair_img svm_model result_binary_lesion_mask [options]

-s<string>                        : path to input image (default: .)

-d<string>                        : path to intermediate result (default: /tmp)

-p<string>                        : path where this package installed

-o<string>                        : path to final result (default: .)

-w<float>,<float>,<float>,<float> : weighting parameter for each modality (default: 1.0,1.0,1.0,1.0)

-t<float>                         : threshold for getting lesion mask (default: 0.0)

where:

t1_img: T1 weight image of testing subject.

t2_img: T2 weight image of testing subject.

pd_img: PD weight image of testing subject.

fl_img: Flair weight image of testing subject.

svm_model: SVM model generated in training stage and will be used to test this subject.

Result_binary_lesion_mask: white matter lesion segmentation result in binary mask format.

-s: path where input images locate.

-d: path where intermediate files locate.

-p: path where the WML segmentation package installed.

-o: path where the segmentation result will be stored.

-w: weighting parameters where 4 modalities (t1, t2, pd, flair) in forming feature vector for testing, default: 1.0,1.0,1.0,1.0

-t: threshold to get final binary lesion mask.

The output file will be “WML.lesion.mask.img” under sub-folder of each subject.

Step 1: Coregistration T1, T2, PD to FLAIR space

Use command “flirt” embedded in FSL to coregister T1, T2, PD to FL space, separately.

Sample command:

flirt –in subjectID.T1.byte.hdr -ref subjectID.FL.byte.hdr -out subjectID.T1.byte.coreg.hdr -omat subjectID.T1.byte.coreg.mat -bins 256 -cost corratio -searchrx -90 90 -searchry -90 90 -searchrz -90 90 -dof 12 -interp trilinear

flirt -in subjectID.T2.byte.hdr -ref subjectID.FL.byte.hdr -out subjectID.T2.byte.coreg.hdr -omat subjectID.T2.byte.coreg.mat -bins 256 -cost normmi -searchrx -90 90 -searchry -90 90 -searchrz -90 90 -dof 12 -interp trilinear

flirt -in subjectID.PD.byte.hdr -ref subjectID.FL.byte.hdr -applyxfm -init subjectID.T2.byte.coreg.mat -out subjectID.PD.byte.coreg.hdr

Step 2: smooth image a little bit

Use command “extractVentricle” to smooth input images a little bit to deal with potential noise.

Sample command:

extractVentricle subjectID.T1.byte.coreg.img subjectID.T1.byte.coreg.smooth.img -b0.5

extractVentricle subjectID.T2.byte.coreg.img subjectID.T2.byte.coreg.smooth.img -b0.5

extractVentricle subjectID.PD.byte.coreg.img subjectID.PD.byte.coreg.smooth.img -b0.5

extractVentricle subjectID.FL.byte.coreg.img subjectID.FL.byte.coreg.smooth.img -b0.5

Step 3: Skull-strip T2,PD,FLAIR images according to brain mask in T1 space

There are 5 sub-steps in this step due to different transformation.

Sub-step 1: generate brain mask in acpc aligned space

Using command “extractVentricle” to generate brain mask in acpc aligned space based on skull-stripped T1. sub-sub-steps in this sub-step include

1). Thresholding skull-stripped T1 to generate a binary brain mask.

2). “erose” the mask a little bit to exclude potential isolated voxels.

3). Using region growing to further exclude isolated regions.

Sample command:

extractVentricle subjectID.T1.byte_cbq.img brain.lao.mask.img -t1

OpeningCloseingRegiongrowing3DBinaryImg brain.lao.mask.img brain.lao.mask.close.img -t1 -r2

OpeningCloseingRegiongrowing3DBinaryImg brain.lao.mask.close.img brain.lao.mask.close.rg.img -t2 -n20000 -m1 -g1658

Sub-step 2: transform brain mask from acpc aligned space to t1 space

Using command "anslice4volrot" and transformation data file “angles.deg”to transform brain mask from aligned space to t1 space.

Sample command:

anslice4volrot -r $angle[0] $angle[1] $angle[2] -b brain.lao.mask.close.rg.open.img brain.lao.mask.t1space

Sub-step 3: transform brain mask from t1 space to flair space

Using command "flirt" and transformation matrix file generated in coregistration step to transform brain mask from t1 space to flair space.

Sample command:

flirt -in brain.lao.mask.t1space.hdr –ref subjected.FL.byte.hdr -applyxfm -init subjected.T1.byte.coreg.mat -out brain.lao.mask.flspace.hdr

Sub-step 4: threshold brain mask in flair space to get actual mask

Using command “extractVentricle” to threshold brain mask in flair space to get actual mask.

Sample command:

extractVentricle brain.lao.mask.flspace.img brain.lao.mask.flspace.threshold.img -t150

Sub-step 5: use actual brain mask in flair space to skull strip all modalities in flair space.

Using command "MaskOutImg" and brain mask generated in sub-step 4 to skull strip all coregistered image in flair space

Sample command:

MaskOutImg subjectID.T1.byte.coreg.smooth.img brain.lao.mask.flspace.threshold.img subjectID].T1.byte.coreg.smooth.brain.img -t1

MaskOutImg subjectID.T2.byte.coreg.smooth.img brain.lao.mask.flspace.threshold.img subjectID].T2.byte.coreg.smooth.brain.img -t1

MaskOutImg subjectID.PD.byte.coreg.smooth.img brain.lao.mask.flspace.threshold.img subjectID].PD.byte.coreg.smooth.brain.img -t1

MaskOutImg subjectID.FL.byte.coreg.smooth.img brain.lao.mask.flspace.threshold.img subjectID].FL.byte.coreg.smooth.brain.img -t1

Step 4: Inhomogeneity corrrection using N3

Using script command "InhomoCorrection_n3.pl" to correct inhomogeneity in all coregistered and skull-stripped images.

Sample commands:

InhomoCorrection_n3.pl subjectID.T1.byte.coreg.smooth.brain.img subjectedID.T1.byte.coreg.smooth.brain.correct.img -i6

InhomoCorrection_n3.pl subjectID.T2.byte.coreg.smooth.brain.img subjectedID.T2.byte.coreg.smooth.brain.correct.img -i6

InhomoCorrection_n3.pl subjectID.PD.byte.coreg.smooth.brain.img subjectedID.PD.byte.coreg.smooth.brain.correct.img -i6

InhomoCorrection_n3.pl subjectID.FL.byte.coreg.smooth.brain.img subjectedID.FL.byte.coreg.smooth.brain.correct.img –i9

Step 5: histogram based intensity normalization

Using "MatchHistogramsOfImgA2ImgB_special" to match histogram of each individual image to the histogram of model image.

Sample command:

MatchHistogramsOfImgA2ImgB_special subjectID.T1.byte.coreg.smooth.brain.correct.img $t1_model $subjectID.T1.byte.coreg.smooth.brain.correct.match.img -s5.0 -t5

MatchHistogramsOfImgA2ImgB_special subjectID.T2.byte.coreg.smooth.brain.correct.img $t2_model $subjectID.T2.byte.coreg.smooth.brain.correct.match.img -s5.0 -t5

MatchHistogramsOfImgA2ImgB_special subjectID.PD.byte.coreg.smooth.brain.correct.img $pd_model $subjectID.PD.byte.coreg.smooth.brain.correct.match.img -s5.0 -t5

MatchHistogramsOfImgA2ImgB_special subjectID.FL.byte.coreg.smooth.brain.correct.img $fl_model subjectID.FL.byte.coreg.smooth.brain.correct.match.img -s5.0 –f

Step 6: Remove eye tissue in skull-stripped FLAIR image

Using command “WMLRemoveEyeInFlair_improve_again” to remove eye tissue is skull-stripped FLAIR image.

Sample command:

WMLRemoveEyeInFlair_improve_again  subjectID.FL.byte.coreg.smooth.brain.correct.match.img subjectID.FL.byte.coreg.smooth.brain.correct.match.noeye.img -i5 -r0.1,0.99

Step 7: voxel-wised testing

Using command “WMLTestSingleImageKeepAllFunctionValue_NoSmooth” and svm model generated in the training stage to perform voxel-wised testing.

Sample command:

WMLTestSingleImageKeepAllFunctionValue_NoSmooth subjectID.T1.byte.coreg.smooth.brain.correct.match.img subjectID.T2.byte.coreg.smooth.brain.correct.match.img subjectID.PD.byte.coreg.smooth.brain.correct.match.img subjectID.FL.byte.coreg.smooth.brain.correct.match.noeye.img $svm_model WML.lesion.mask.img.allvalue -m1 -r2

Step 8: Threshold the evaluation image & false positive elimination

Using command “OneImgOp” is threshold evaluation map from svm testing and eliminates potential false positives.

Sample command:

OneImgOp WML.lesion.mask.img.allvalue.invert WML.lesion.mask.img -t$lesion_threshold -m0 -f1

## USAGE

- Executing:

It is very simple. There are actually 2 stand alone scripts, one for training (WMLSegmentation_train.pl) and the other for testing (WMLSegmentation_test.pl). Run either command (WMLSegmentation_train.pl or WMLSegmentation_test.pl) without any option, a brief usage description will be displayed.

Training:

Usage: WMLSegmentation_train.pl train_list model [options]

-d<int>,<int>,<int>  : dimension of input images in XY plane (default: 256,256,46)

-v<float>,<float>,<float>  : image resolution (default: 0.9375,0.9375,3.0)

-t<float>,<float>    : threshold for TPVF and FPVF (default: 0.9,0.1)

-i<int>              : maximum iteration number (default: 10)

-r<int>              : neighbor size (default: 1)

-M<int>              : use premask or not 0: not use 1: use (default: not use)

-p<string>           : path where this package installed

-P<string>           : path where to put intermediate result (default: /tmp)

-w <float>,<float>,<float>,<float> : weighting parameters for t1, t2, pd, flair respectively (default: 1.0,1.0,1.0,1.0)

Usage: WMLSegmentation_test.pl t1_img t2_img pd_img flair_img svm_model result_binary_lesion_mask [options]

-s<string>                        : path to input image (default: .)

-d<string>                        : path to intermediate result (default: /tmp)

-p<string>                        : path where this package installed

-o<string>                        : path to final result (default: .)

-w<float>,<float>,<float>,<float> : weighting parameter for each modality (default: 1.0,1.0,1.0,1.0)

-t<float>                         : threshold for getting lesion mask (default: 0.0)

There are some conventions in order to smoothly run this program, they are:

1. The expected format of the images is Analyze unsigned char with image size of 256 c 256 x z, where z is the number of slices. Input images should always come along with a header file.

2. Current version of protocol is compiled and executed under UNIX (athenas) systems and need to recompile and retest if exported to Linux system.

3. There are other two files need to be existed. i.e. angles.deg and skull-stripped T1 image (subjectID.T1.byte_cbq.img and subjectID.T1.byte_cbq.hdr)

### Troubleshooting

- If the program gets interrupted, please check whether you provide lesion mask while running. For subject with high lesion loads, the label correction program may fail if not specify lesion mask.

- If the program gets interrupted, please also check if the input image is with BYTE format, the program will only deal with 8 bits/voxel input image and only output 8 bits/voxel output image.

- Since the protocol assume to use Jacob brain as template, so the model size has been hard coded to be 256 x 256 x 198. If you want to use some other template, the script may fail due to image size issue. Please make sure you are using Jacob brain as template before you run the protocol. If you want to use other template, please modify the script accordingly.

## REFERENCES

[1] ZQ Lao, DG Shen, A Jawad, B Karacali, DF Liu, ER Melhem, NR Bryan, C Davatzikos, "Automated Segmentation of White Matter Lesions in 3D Brain MR Images, Using Multivariate Pattern Classification", Third IEEE International Symposium on Biomedical Imaging (ISBI 2006), April 6-9, 2006, Arlington, VA, USA.

[2] G. Moonis, Zhiqiang Lao, D. Liu, C. Davatzikos, R.N. Bryan, A. Jawad, "Validation of an automated segmentation technique against at established visual rating system for measures of brain atrophy and white matter lesion load", ASNR 44th annual meeting, San Diego, California, April 29-May 5, 2006.

[3] ZQ Lao, DG Shen, DF Liu, AF Jawad, ER Melhem, LJ Launer, RN Bryan, C. Davatzikos, "Computer-assisted segmentation of white matter lesions in 3D MR images, using support vector machine", accepted by academic radiology.

## OS support

Right now, this software only works on Athena machines. If you want to run it under Linux clusters, you will need to re-compile the software package under Linux environment.

## Quick Start Guide

This is a single command software. Using the information described in the section of "Usage", it is then straightforward to get a quick start.

## Use of building tools

"make" and "gcc" are required to build the executable binary.

## Verification of results

At current stage, only visual inspection is available to check the result.

## Contacts

Lao, Zhiqiang <Zhiqiang.Lao@uphs.upenn.edu>